ROCKFORD TOWNSHIP DRIVEWAY ACCESS POLICY

*	These design standards have been prepared in the interest of safety and public welfare as a guide to public agencies and private owners in preparing plans and applications for entrances and exits to and from township roads.

*	Application for permission to construct entrance or exit driveways along township roads shall be made on forms supplied by Rockford Township and shall be submitted to the Clerk’s Office.

DESIGN

DRIVEWAYS shall be at right angles to the township roads for the total designed right-of-way width.

MINIMUM SPACING between driveway culvert ends shall be 16 feet.

MINIMUM DISTANCE along shoulder from driveway to projected right-of-way line of intersecting road, highway, or railroad, 75 feet.

RESIDENCE DRIVEWAYS shall not exceed 16 feet in width measured at right angles to the centerline of the driveway.

SIGHT DISTANCE – 300’, except in residential and platted areas.

The above-specified distances are minimums.  Each excess request will be reviewed on its own merits to determine if these distances are adequate.

Note: Use 1:4 Side Slope

Rural residential Entrance: 16’

Farm Entrance: 20’

Field Entrance: 20’

Commercial: 30’

(Drawing can be viewed at the Town Hall Office)

CONSTRUCTION

- All work done on township right-of-way is subject to approval by the Staff.

- Driveway shall be so constructed as to slope down and away from the shoulder line of the road for a distance of at least 10’ with a fall of at least 8” inches.

- Side slopes shall not be constructed steeper than 4 feet horizontally to one foot vertically (4:1), unless approved by the staff.  The slopes shall be finished by seeding or sod.

- Proper and adequate drainage facilities shall be provided as required by the Township.  The owner shall pay all construction costs including such cost of drainage structures as may be determined by Rockford Township Resolution.

- The area on road right-of-way other than that of the driveway itself will normally be in the form of a ditch.  Modifications may be permitted, provided proper drainage is furnished and curbs, hedges, or other approved safeguards are installed to prevent misuse.  Proper authority for modifications must be obtained from the Staff before any work is started.

REQUIREMENTS

- No work under this application is to be started until application is approved and the permit issued.

- Where work on traveled roadway is necessary, traffic must be protected, and flags, flares, and proper barricades must be placed in accordance with the standards of the Minnesota Department of Transportation.

- No foreign materials such as dirt, gravel, concrete or bituminous material shall be left or deposited on the road during construction of the driveway or installation of drainage facilities.

- Roadside and right-of-way must be cleaned up after work is completed.

- After driveway construction is completed, the applicant shall notify the Clerk’s Office that the work has been completed and is ready for final inspection and approval by the Board.

- No changes or alterations in entrances may be made at any time without permission from the Township.

- Failure to comply with the conditions of this Access Permit will result in penalties described in the resolution sited above.

IMPROVEMENTS

- Hard surfacing must be back at least 8 feet from the edge of the road.

- Normal maintenance, example adding new gravel, is not considered an improvement.

IMPORTANT NOTE: ALL CULVERT(S) MUST BE NEW, STEEL, AND A MINIMUM OF 15” DIAMETER WITH APRONS.