ORDINANCE # 2010.2

AN ORDINANCE REGULATING MAILBOX INSTALLATION ADJACENT TO TOWNSHIP ROADS

The Town Board of Rockford Township does hereby ordain:

- The purpose of this ordinance is to regulate the existing and new mailbox installations within the township right-of-way.

- Rockford Township incorporates by reference Minn. Stat. 169.072, Unauthorized Mailbox Installation as may be amended from time to time.  See attached statute.

- Along with the requirements of statute the Township will include the following requirements for mailbox installations:

- Meets all US Postal Service requirements.

- No more than a maximum weight of fifteen (15) pounds attached to the mailbox support.

- The height of the mailbox and newspaper tubes will be 42” to 46” above the edge of the roadway.

- The mailbox front will be six inches (6”) from the shoulder of the roadway.

- Maximum of two (2) mailboxes and two (2) newspaper tubes per installation, newspaper tube(s) shall be installed together with mailbox (es) wherever practical.

- A maximum of twenty-four inch (24”) cross arm length, if used.  If a swing-away mailbox support is used this item shall not apply.

- Maximum dimensions of mounting post shall be 4”X4” (wood), 4 ½” diameter (wood), 2” diameter (metal pipe) (with a weight not in excess of 2.6 lbs. per foot) or a sign post if a swing away mailbox installation is used.  If more than one support is needed the supports will be no closer than three (3) feet apart.

- No support structure shall be embedded in the ground more than twenty-four inches (24”).  No concrete shall be used to anchor the mailbox support.

- The placement of heavy or large object as standards or bases is not permitted.

- Violation of this ordinance shall be a misdemeanor, as stated in 169.072 subd. 3, punishable in accordance with state law.

- This ordinance becomes effective immediately upon its passage and publication according to law.

Passed this 2 day of March 2010 by the Rockford Town Board.

Dennis Beise

Rockford Town Board, Chairperson

Attest:

Rachelle L. McDougall

Clerk/Treasurer